МИНИСТЕРСТВО ОБРАЗОВАНИЯ И НАУКИ

РОССИЙСКОЙ ФЕДЕРАЦИИ

ФЕДЕРАЛЬНОЕ ГОСУДАРСТВЕННОЕ БЮДЖЕТНОЕ ОБРАЗОВАТЕЛЬНОЕ УЧРЕЖДЕНИЕ

ВЫСШЕГО ОБРАЗОВАНИЯ

«БЕЛГОРОДСКИЙ ГОСУДАРСТВЕННЫЙ

ТЕХНОЛОГИЧЕСКИЙ УНИВЕРСИТЕТ им. В.Г.ШУХОВА»

(БГТУ им. В.Г. Шухова)

Кафедра программного обеспечения вычислительной техники и

автоматизированных систем

Лабораторная работа №1

дисциплина: ИРТСиК

тема: «Фильтр Калмана »

Белгород 2023

Фи́льтр Ка́лмана — эффективный рекурсивный фильтр, оценивающий вектор состояния динамической системы, используя ряд неполных и зашумленных измерений.

Задание: рассмотрим двухосную тележку с ведущими мотор-колесами на одной из осей (тележка симметрична), передвигающуюся по ровной гладкой поверхности, на которой имеются области с пониженными коэффициентами трения качения (μ_c) и скольжения (μ_f). Управляющая система может контролировать крутящие моменты на колесах (m_1,m_2). Тележка имеет известные геометрические параметры: длина, ширина, масса, колесная база, колея, радиус колес, ширина колес (все колеса одинаковы). Колеса тележки не поворачиваются вокруг вертикальной оси.

Задача: провести тележку по заданной желаемой траектории, не имея карты расположения зон сниженного трения.

Main.py

import sys

from PyQt6.QtWidgets import QApplication

from scene import MainWindow

app = QApplication(sys.argv)

window = MainWindow()
window.show()

app.exec()

settings.py

MAP_WIDTH = 1080
MAP_HEIGHT = 800

CAR_LENGTH = 50
CAR_WIDTH = 25
WHEEL_LENGTH = 15
WHEEL_WIDTH = 7
START_POSITION_CAR = (CAR_WIDTH + WHEEL_WIDTH * 2, MAP_HEIGHT - CAR_LENGTH - WHEEL_LENGTH * 2)

GRADIENT_RESISTANCE_FILL = True

FPS = 60
FRAME_TIME = 1000. / FPS

car.py

from PyQt6.QtCore import QPointF
from PyQt6.QtGui import QMatrix4x4, QPainter, QColor

from math import pi, cos, sin

from map import Map
from settings import START_POSITION_CAR, CAR_WIDTH, CAR_LENGTH, WHEEL_WIDTH, WHEEL_LENGTH


class Car:
    t = 1
    MAP_DIV = 100

    MOMENT_STEP = 1
    MOMENT_REDUCTION_STEP = MOMENT_STEP / 2
    MAX_M = 350
    FRICTION_CONTACT_RADIUS = WHEEL_WIDTH / 2

    # Масса машины
    M = 1000
    # Масса колеса
    W = 20
    # Радиус колеса
    R = 0.5
    # Ширина колеи
    L = 2.5

    G = 9.8

    def __init__(self, map: Map, trace_color: str, with_abs: bool = False):
        self.map = map
        self.trace_color = trace_color
        self.with_abs = with_abs
        self.body, self.wheels, self.left_wheel_center, self.right_wheel_center = self.__init_model()

        self.ml, self.mr = 0, 0
        self.x, self.y = START_POSITION_CAR
        self.x *= self.MAP_DIV
        self.y *= self.MAP_DIV
        self.alfa = 0

        self.vfl, self.vfr, self.vbl, self.vbr = 0, 0, 0, 0
        self.ipsilon = 0
        self.u = 0
        self.w = 0

        # 0 - покой
        # 1 - скольжение
        # 2 - качение
        self.l_state = 0
        self.r_state = 0

    def _moment_reduction(self):
        self.ml = 0
        self.mr = 0

    def next_x(self):
        return self.x + self.t * self.u * cos(self.ipsilon)

    def next_y(self):
        return self.y + self.t * self.u * sin(self.ipsilon)

    def next_alfa(self):
        return self.alfa + self.t * self.w

    def _get_uk(self, center=None):
        if center is None:
            projected_center = (
                        self.get_project_matrix() * ((self.right_wheel_center + self.left_wheel_center) / 2)).toPoint()
        else:
            projected_center = (self.get_project_matrix() * center).toPoint()

        return self.map.get_resistance(projected_center.x(), projected_center.y())

    def _get_I(self, m=None, center=None):
        if m is None:
            m = (self.ml + self.mr) / 2
        uk = self._get_uk(center)
        k = 1
        if uk * (self.M / 4 + self.W) * self.R * self.G < m:
            k = uk + 0.5
        if self.with_abs:
            k = 1
        return (k * self.M / 4 + self.W / 2) * self.R ** 2

    def _get_mk(self, center=None):
        uk = self._get_uk(center)
        if self.with_abs:
            uk = 0.5
        return self.M / 4 * self.G * self.R * (uk + 0.7)

    @staticmethod
    def _sign(value):
        if value > 0:
            return 1
        if value < 0:
            return -1
        return 0

    def next_vbl(self):
        new_vbl = self.vbl + (self.ml - self._sign(self.vbl) * self._get_mk(self.left_wheel_center)) \
                  / self._get_I(self.ml, self.left_wheel_center) \
                  * self.t
        if new_vbl * self.vbl < 0:
            return 0
        return new_vbl

    def next_vbr(self):
        new_vbr = self.vbr + (self.mr - self._sign(self.vbr) * self._get_mk(self.right_wheel_center)) \
                  / self._get_I(self.mr, self.right_wheel_center) \
                  * self.t
        if new_vbr * self.vbr < 0:
            return 0
        return new_vbr

    def next_impsilon(self):
        return self.ipsilon + (self.vbl - self.vbr) * self.t * self.R / (2 * pi * self.L)

    def next_u(self):
        new_u = self.u + self.R * self.t * \
                (self.vbr if abs(self.vbl) > abs(self.vbr) else self.vbl)

        # TODO
        if abs(self.u):
            new_u -= \
                self._sign(self.u) * self._get_mk() / self._get_I() * self.t / 100

        if new_u * self.u < 0:
            self.w = 0
            return 0
        return new_u

    def next_w(self, prev_ipsilon):
        new_w = self.w + (self.ipsilon - prev_ipsilon) / self.t

        # TODO
        if abs(self.w):
            new_w -= self._sign(self.w) * 0.1

        if new_w * self.w < 0:
            self.ipsilon = self.alfa
            return 0
        return new_w

    def step(self):
        self.x = self.next_x()
        self.y = self.next_y()
        self.alfa = self.next_alfa()

        self.vbl = self.next_vbl()
        self.vbr = self.next_vbr()

        prev_ipsilon = self.ipsilon
        self.ipsilon = self.next_impsilon()

        self.u = self.next_u()
        self.w = self.next_w(prev_ipsilon)

        self.map.set_trace(
            self.get_center(),
            self.trace_color
        )
        self._moment_reduction()

    def get_project_matrix(self) -> QMatrix4x4:
        a = QMatrix4x4()
        a.setToIdentity()
        a.rotate(self.alfa * 180 / pi, 0, 0, 1)

        t = QMatrix4x4()
        t.setToIdentity()
        t.translate(self.x / self.MAP_DIV, self.y / self.MAP_DIV)
        return t * a

    @staticmethod
    def __init_model():
        rotate = QMatrix4x4()
        rotate.setToIdentity()
        rotate.rotate(90, 0, 0, 1)

        def get_polygon(x, y, w, l) -> [QPointF]:
            return [
                rotate * QPointF(x, y),
                rotate * QPointF(x + w, y),
                rotate * QPointF(x + w, y + l),
                rotate * QPointF(x, y + l),
            ]

        yield get_polygon(
            -CAR_WIDTH / 2,
            -CAR_LENGTH / 2,
            CAR_WIDTH,
            CAR_LENGTH
        )
        yield [
            get_polygon(
                -CAR_WIDTH / 2 - WHEEL_WIDTH,
                -CAR_LENGTH / 2 - WHEEL_LENGTH / 2,
                WHEEL_WIDTH,
                WHEEL_LENGTH
            ),
            get_polygon(
                CAR_WIDTH / 2,
                -CAR_LENGTH / 2 - WHEEL_LENGTH // 2,
                WHEEL_WIDTH,
                WHEEL_LENGTH
            ),
            get_polygon(
                CAR_WIDTH / 2,
                CAR_LENGTH / 2 - WHEEL_LENGTH / 2,
                WHEEL_WIDTH,
                WHEEL_LENGTH
            ),
            get_polygon(
                -CAR_WIDTH / 2 - WHEEL_WIDTH,
                CAR_LENGTH / 2 - WHEEL_LENGTH / 2,
                WHEEL_WIDTH,
                WHEEL_LENGTH
            )
        ]
        yield rotate * QPointF(
            -CAR_WIDTH / 2 - WHEEL_WIDTH / 2,
            CAR_LENGTH / 2
        )
        yield rotate * QPointF(
            CAR_WIDTH / 2 + WHEEL_WIDTH / 2,
            CAR_LENGTH / 2
        )

    def map_point(self, matrix: QMatrix4x4, point: QPointF) -> QPointF:
        return matrix * point

    def map_polygon(self, matrix: QMatrix4x4, points: [QPointF]) -> [QPointF]:
        return list(
            map(
                lambda point: self.map_point(matrix, point),
                points
            )
        )

    def draw(self, painter: QPainter):
        matrix = self.get_project_matrix()

        painter.setBrush(QColor(self.trace_color))
        painter.drawPolygon(
            self.map_polygon(matrix, self.body)
        )

        painter.setBrush(QColor('black'))
        painter.drawPolygon(
            self.map_polygon(matrix, self.wheels[0])
        )
        painter.drawPolygon(
            self.map_polygon(matrix, self.wheels[1])
        )

        painter.setBrush(QColor('blue'))
        painter.drawPolygon(
            self.map_polygon(matrix, self.wheels[2])
        )
        painter.drawPolygon(
            self.map_polygon(matrix, self.wheels[3])
        )

        painter.setBrush(QColor('red'))
        painter.drawEllipse(
            self.map_point(matrix, self.left_wheel_center),
            self.FRICTION_CONTACT_RADIUS,
            self.FRICTION_CONTACT_RADIUS
        )
        painter.drawEllipse(
            self.map_point(matrix, self.right_wheel_center),
            self.FRICTION_CONTACT_RADIUS,
            self.FRICTION_CONTACT_RADIUS
        )

    def forward(self):
        self.ml = self.MAX_M
        self.mr = self.MAX_M

    def back(self):
        self.ml = -self.MAX_M
        self.mr = -self.MAX_M

    def right(self):
        self.mr = -self.MAX_M / 10

    def left(self):
        self.ml = - self.MAX_M / 10

    def get_center(self) -> QPointF:
        return self.get_project_matrix() * QPointF(0., 0.)

map.py

from PyQt6.QtCore import QPointF, QPoint
from PyQt6.QtGui import QImage, QPixmap, QColor, QPainter

from settings import MAP_WIDTH, MAP_HEIGHT, GRADIENT_RESISTANCE_FILL, WHEEL_LENGTH


class Map:
    BRUSH_RADIUS = 50
    BRUSH_STEP = 0.1

    RESISTANCE_RANGE = 1.0
    RESISTANCE_CENTER = 0.0

    def __init__(self):
        self.break_points = []
        self.w = MAP_WIDTH
        self.h = MAP_HEIGHT
        self._map = {
            x: ({
                y: self.RESISTANCE_CENTER for y in range(self.h)
            })
            for x in range(self.w)
        }
        self._image = QImage(MAP_WIDTH, MAP_HEIGHT, QImage.Format.Format_ARGB32)
        if GRADIENT_RESISTANCE_FILL:
            self.init_gradient()

    def set_resistance(self, x: int, y: int, resistance: float):
        if resistance > self.RESISTANCE_RANGE:
            resistance = self.RESISTANCE_RANGE
        if resistance < -self.RESISTANCE_RANGE:
            resistance = -self.RESISTANCE_RANGE

        self._map[x][y] = resistance

        if resistance > 0:
            green_part = 255 - 255 * resistance / self.RESISTANCE_RANGE
            red_part = 255
        else:
            green_part = 255
            red_part = 255 + 255 * resistance / self.RESISTANCE_RANGE

        self._image.setPixel(x, y, QColor(int(red_part), int(green_part), 0).rgb())

    def get_resistance(self, x: int, y: int) -> float:
        try:
            return self._map[x][y] + self.RESISTANCE_RANGE / 2
        except KeyError:
            return 0

    def get_pixmap(self) -> QPixmap:
        return QPixmap.fromImage(self._image)

    def init_gradient(self):
        x_inc = self.RESISTANCE_RANGE * 2 / self.w
        y_inc = self.RESISTANCE_RANGE * 2 / self.h

        x_res = -self.RESISTANCE_RANGE
        for x in range(self.w):
            y_res = -self.RESISTANCE_RANGE
            for y in range(self.h):
                self.set_resistance(x, y, y_res)
                y_res += y_inc
            x_res += x_inc

    def init_zero(self):
        for x in range(self.w):
            for y in range(self.h):
                self.set_resistance(x, y, 0)

    def brush(self, x: int, y: int):
        start_x = 0 if x < self.BRUSH_RADIUS else x - self.BRUSH_RADIUS
        end_x = self.w if x + self.BRUSH_RADIUS > self.w else x + self.BRUSH_RADIUS

        start_y = 0 if y < self.BRUSH_RADIUS else y - self.BRUSH_RADIUS
        end_y = self.h if y + self.BRUSH_RADIUS > self.h else y + self.BRUSH_RADIUS
        for x in range(start_x, end_x):
            for y in range(start_y, end_y):
                resistance = self.get_resistance(x, y)
                if (resistance + self.BRUSH_STEP) > self.RESISTANCE_RANGE:
                    self.set_resistance(x, y, -self.RESISTANCE_RANGE)
                else:
                    self.set_resistance(x, y, resistance + self.BRUSH_STEP)

    def set_trace(self, point: QPointF, color: str = 'black'):
        self._image.setPixel(
            point.toPoint(),
            QColor(color).rgb()
        )

    def paint_break_points(self, painter: QPainter):
        painter.setBrush(QColor('purple'))
        for break_point in self.break_points:
            painter.drawEllipse(
                break_point,
                WHEEL_LENGTH,
                WHEEL_LENGTH
            )

    def add_break_point(self, point: QPoint):
        self.break_points.append(point)

    def clean_break_points(self):
        self.break_points = []

kalman.py

from random import random

from map import Map
from trace import TraceCar


class KalmanCar(TraceCar):
    k = 0.9
    SENSOR_VARIANCE = 0.05 ** 2
    FRICTION_DIRECTION_VARIANCE = 0.5 ** 2
    FRICTION_ANGEL_VARIANCE = 0.5 ** 2

    def __init__(self, map: Map, trace_color: str):
        super().__init__(map, trace_color)
        self.Ekvbl = self.FRICTION_DIRECTION_VARIANCE
        self.Ekvbr = self.FRICTION_DIRECTION_VARIANCE

        self.Ekipsilon = self.FRICTION_ANGEL_VARIANCE
        self.Eku = self.FRICTION_DIRECTION_VARIANCE
        self.Ekw = self.FRICTION_ANGEL_VARIANCE

    def kalman(self, Xnew, Xsensor, variance, Ek):
        Ek = self.SENSOR_VARIANCE * (Ek + variance) / (Ek + variance + self.SENSOR_VARIANCE)
        k = Ek / self.SENSOR_VARIANCE
        return k * Xsensor + (1 - k) * Xnew, Ek

    @staticmethod
    def get_sensor_data(value, variance=None):
        if variance is None:
            variance = KalmanCar.SENSOR_VARIANCE
        if value:
            return value + (random() * variance * 2 - variance)
        return value

    def next_vbl(self):
        self.with_abs = True
        abs_vbl = self.get_sensor_data(super().next_vbl())
        self.with_abs = False
        vbl = super().next_vbl()

        new_vbl, self.Ekvbl = self.kalman(vbl, abs_vbl, self.FRICTION_DIRECTION_VARIANCE, self.Ekvbl)
        return new_vbl

    def next_vbr(self):
        self.with_abs = True
        abs_vbr = self.get_sensor_data(super().next_vbr(), 1e-23)
        self.with_abs = False
        vbr = super().next_vbr()

        new_vbr, self.Ekvbr = self.kalman(vbr, abs_vbr, self.FRICTION_DIRECTION_VARIANCE, self.Ekvbr)
        return new_vbr

    def next_u(self):
        self.with_abs = True
        abs_u = self.get_sensor_data(super().next_u())
        self.with_abs = False
        u = super().next_u()

        new_u, self.Eku = self.kalman(u, abs_u, self.FRICTION_DIRECTION_VARIANCE, self.Eku)
        return new_u


    def next_w(self, prev):
        self.with_abs = True
        abs_w = self.get_sensor_data(super().next_w(prev))
        self.with_abs = False
        w = super().next_w(prev)

        new_w, self.Ekw = self.kalman(w, abs_w, self.FRICTION_DIRECTION_VARIANCE, self.Ekw)
        return new_w

trace.py

from car import Car
from map import Map


class TraceCar(Car):

    def __init__(self, map: Map, trace_color: str, with_abs: bool = False):
        super().__init__(map, trace_color, with_abs)
        self.steps = \
            ['left'] * 183 + \
            [''] * 100 + \
            ['forward'] * 200 + \
            [''] * 80 + \
            ['right'] * 365 + \
            [''] * 80 + \
            ['forward'] * 50 + \
            [''] * 130 + \
            ['left'] * 365 + \
            [''] * 100 + \
            ['forward'] * 170 + \
            [''] * 80 + \
            ['right'] * 365 + \
            [''] * 80 + \
            ['forward'] * 30

    def step(self):
        super().step()
        if not self.steps:
            return

        step = self.steps[0]
        if step == 'forward':
            self.forward()
        elif step == 'back':
            self.back()
        elif step == 'left':
            self.left()
        elif step == 'right':
            self.right()

        self.steps = self.steps[1:]

scene.py

import typing
from datetime import datetime

from PyQt6 import QtGui
from PyQt6.QtCore import QRect, QPoint, Qt, QRectF, QThread, QEventLoop
from PyQt6.QtGui import QPainter, QBrush, QPen, QColor, QPixmap
from PyQt6.QtWidgets import QMainWindow


from car import Car
from kalman import KalmanCar
from map import Map
from settings import MAP_WIDTH, MAP_HEIGHT, FRAME_TIME
from trace import TraceCar


class MainWindow(QMainWindow):

    def __init__(self):
        super().__init__()

        self.setWindowTitle("Модель тележки")
        self.setFixedWidth(MAP_WIDTH)
        self.setFixedHeight(MAP_HEIGHT)

        self.map = Map()
        self.cars = []

        self.run = False
        self.active_events = {
            'forward': False,
            'right': False,
            'left': False,
            'back': False
        }

    def activate_event(self, event: str):
        self.active_events[event] = True

    def deactivate_event(self, event: str):
        self.active_events[event] = False

    def paint_resistance(self, painter: QPainter):
        pixmap = self.map.get_pixmap()
        painter.drawTiledPixmap(
            QRect(0, 0, MAP_WIDTH, MAP_HEIGHT),
            pixmap,
            QPoint(0, 0)
        )

    def paintEvent(self, e):
        painter = QPainter(self)
        self.paint_resistance(painter)
        self.map.paint_break_points(painter)
        for car in self.cars:
            car.draw(painter)

    def mouseReleaseEvent(self, mouse_event: typing.Optional[QtGui.QMouseEvent]) -> None:
        point = mouse_event.pos()
        button = mouse_event.button()

        if button is Qt.MouseButton.LeftButton:
            self.map.brush(point.x(), point.y())
        elif button is Qt.MouseButton.RightButton:
            self.map.add_break_point(point)

        self.update()

    def start(self):
        self.run = True
        prev_frame = datetime.now()
        while self.run:
            if (datetime.now() - prev_frame).microseconds < FRAME_TIME:
                continue
            prev_frame = datetime.now()
            for car in self.cars:
                car.step()
                if self.active_events['forward']:
                    car.forward()

                if self.active_events['left']:
                    car.left()

                if self.active_events['right']:
                    car.right()

                if self.active_events['back']:
                    car.back()

            self.update()
            QEventLoop().processEvents(QEventLoop.ProcessEventsFlag.AllEvents)

    def keyPressEvent(self, a0: typing.Optional[QtGui.QKeyEvent]) -> None:
        key = a0.key()
        if self.run:
            if key == 16777234:
                self.activate_event('left')
            elif key == 16777235:
                self.activate_event('forward')
            elif key == 16777236:
                self.activate_event('right')
            elif key == 16777237:
                self.activate_event('back')
        # r
        if key == 82:
            self.cars.append(
                Car(self.map, 'red', True)
            )
            self.update()
        # t
        if key == 84:
            self.cars.append(
                TraceCar(self.map, 'black', True)
            )
            self.update()
        # y
        if key == 89:
            self.cars.append(
                TraceCar(self.map, 'blue')
            )
            self.update()
        # u
        if key == 85:
            self.cars.append(
                KalmanCar(self.map, 'purple')
            )
            self.update()
        # z
        if key == 90:
            self.cars = []
        # space
        if key == 32:
            self.start()
        # b
        if key == 66:
            self.map.clean_break_points()

    def keyReleaseEvent(self, a0: typing.Optional[QtGui.QKeyEvent]) -> None:
        key = a0.key()
        if key == 16777234:
            self.deactivate_event('left')
        elif key == 16777235:
            self.deactivate_event('forward')
        elif key == 16777236:
            self.deactivate_event('right')
        elif key == 16777237:
            self.deactivate_event('back')

    def closeEvent(self, a0: typing.Optional[QtGui.QCloseEvent]) -> None:
        self.run = False